| Pick List 20F2071 |  |  |  |
| --- | --- | --- | --- |
| Line No | Field prompt | Pick list data | Help keyword |
| 011 | BODY OR MOUNTING ASSEMBLY | SECTION TITLE |  |
| 012 | Body/Assembly type | flanged insertion |  |
| 012 | Body/Assembly type | high pressure hot tap |  |
| 012 | Body/Assembly type | in-line pipe spool |  |
| 012 | Body/Assembly type | low pressure hot tap |  |
| 012 | Body/Assembly type | meter run |  |
| 012 | Body/Assembly type | packing gland assembly |  |
| 013 | Sensor end support style | NA |  |
| 013 | Sensor end support style | weld fitting with cap |  |
| 013 | Sensor end support style | weld fitting with plug |  |
| 013 | Sensor end support style | WN flange assembly |  |
| 014a | Process conn nominal size | 1 1/2 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 1 1/4 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 1 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 1/2 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 12.5 mm | NOMINAL SIZE |
| 014a | Process conn nominal size | 19 mm | NOMINAL SIZE |
| 014a | Process conn nominal size | 2 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 3 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 3/4 in | NOMINAL SIZE |
| 014a | Process conn nominal size | 31 mm | NOMINAL SIZE |
| 014a | Process conn nominal size | DN 25 | NOMINAL SIZE |
| 014a | Process conn nominal size | DN 40 | NOMINAL SIZE |
| 014a | Process conn nominal size | DN 50 | NOMINAL SIZE |
| 014a | Process conn nominal size | DN 80 | NOMINAL SIZE |
| 014b | Process conn nom rating | cl 150 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | cl 1500 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | cl 2500 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | cl 300 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | cl 600 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | cl 900 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | PN 100 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | PN 16 | NOMINAL PRESSURE/RATING |
| 014b | Process conn nom rating | PN 40 | NOMINAL PRESSURE/RATING |
| 015a | Process conn termn type | flanged | END CONNECTION CATEGORY |
| 015a | Process conn termn type | threaded | END CONNECTION CATEGORY |
| 015a | Process conn termn type | welded | END CONNECTION CATEGORY |
| 015b | Process conn termn style | BW | END CONNECTION TYPE |
| 015b | Process conn termn style | NTP(M) | END CONNECTION TYPE |
| 015b | Process conn termn style | WN | END CONNECTION TYPE |
| 016a | End support conn nom size | 1 in |  |
| 016a | End support conn nom size | 1/2 in |  |
| 016b | End support conn style | NPT(F) |  |
| 016b | End support conn style | RTJ |  |
| 016b | End support conn style | SW |  |
| 017 | Pipe spool schedule | 40 |  |
| 017 | Pipe spool schedule | 80 |  |
| 017 | Pipe spool schedule | NA |  |
| 018 | Mounting fitting type | 3000 lb thread-o-let |  |
| 018 | Mounting fitting type | NA |  |
| 018 | Mounting fitting type | saddle clamp |  |
| 019 | Body/Assembly material | 316 SST | MATERIAL |
| 019 | Body/Assembly material | aluminum | MATERIAL |
| 019 | Body/Assembly material | carbon steel | MATERIAL |
| 019 | Body/Assembly material | cast iron | MATERIAL |
| 019 | Body/Assembly material | chrome-moly grade F-11 | MATERIAL |
| 019 | Body/Assembly material | chrome-moly grade F-22 | MATERIAL |
| 019 | Body/Assembly material | chrome-moly grade F-91 | MATERIAL |
| 019 | Body/Assembly material | copper | MATERIAL |
| 019 | Body/Assembly material | CPVC | MATERIAL |
| 019 | Body/Assembly material | CPVC | MATERIAL |
| 019 | Body/Assembly material | fiberglass | MATERIAL |
| 019 | Body/Assembly material | PVC plastic | MATERIAL |
| 020 | Packing material | graphite | MATERIAL |
| 020 | Packing material | NA | MATERIAL |
| 021 | Mounting fitting matl | 316 SST | MATERIAL |
| 021 | Mounting fitting matl | carbon steel | MATERIAL |
| 021 | Mounting fitting matl | chrome-moly grade F-11 | MATERIAL |
| 021 | Mounting fitting matl | chrome-moly grade F-22 | MATERIAL |
| 021 | Mounting fitting matl | chrome-moly grade F-91 | MATERIAL |
| 021 | Mounting fitting matl | NA | MATERIAL |
| 022a | Defined attribute 1 name | Bolting material | DEFINABLE ATTRIBUTE NAME |
| 022a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 022b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 023a | Defined attribute 2 name | Gasket type | DEFINABLE ATTRIBUTE NAME |
| 023a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 023b | Defined attribute 2 data | flexitallic | DEFINABLE ATTRIBUTE VALUE |
| 023b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 024 | SENSING ELEMENT | SECTION TITLE |  |
| 025 | Sensor type | diamond shape |  |
| 025 | Sensor type | round cylinder |  |
| 025 | Sensor type | streamline shape |  |
| 026 | Nominal line size | 10 in |  |
| 026 | Nominal line size | 12 in |  |
| 026 | Nominal line size | 14 in |  |
| 026 | Nominal line size | 16 in |  |
| 026 | Nominal line size | 18 in |  |
| 026 | Nominal line size | 2 1/2 in |  |
| 026 | Nominal line size | 2 in |  |
| 026 | Nominal line size | 20 in |  |
| 026 | Nominal line size | 24 in |  |
| 026 | Nominal line size | 3 1/2 in |  |
| 026 | Nominal line size | 3 in |  |
| 026 | Nominal line size | 30 in |  |
| 026 | Nominal line size | 36 in |  |
| 026 | Nominal line size | 4 in |  |
| 026 | Nominal line size | 42 in |  |
| 026 | Nominal line size | 48 in |  |
| 026 | Nominal line size | 5 in |  |
| 026 | Nominal line size | 6 in |  |
| 026 | Nominal line size | 60 in |  |
| 026 | Nominal line size | 72 in |  |
| 026 | Nominal line size | 8 in |  |
| 027 | Sensor nominal size | 1 in |  |
| 027 | Sensor nominal size | 1/2 in |  |
| 027 | Sensor nominal size | 10 |  |
| 027 | Sensor nominal size | 15/16 |  |
| 027 | Sensor nominal size | 2 1/2 in |  |
| 027 | Sensor nominal size | 2 3/8 in OD |  |
| 027 | Sensor nominal size | 25/26 |  |
| 027 | Sensor nominal size | 3/8 in |  |
| 027 | Sensor nominal size | 35/36 |  |
| 027 | Sensor nominal size | 45/46 |  |
| 027 | Sensor nominal size | 5/16 in |  |
| 028 | Temperature element type | 100 Ohm Pt RTD |  |
| 028 | Temperature element type | NA |  |
| 029 | Sensor material | 316 SST | MATERIAL |
| 029 | Sensor material | 316L SST | MATERIAL |
| 029 | Sensor material | carpenter 20 | MATERIAL |
| 029 | Sensor material | hastelloy C | MATERIAL |
| 029 | Sensor material | hastelloy C-276 | MATERIAL |
| 030a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 030a | Defined attribute 1 name | Surface finish | DEFINABLE ATTRIBUTE NAME |
| 030b | Defined attribute 1 data | Cooling type | DEFINABLE ATTRIBUTE VALUE |
| 030b | Defined attribute 1 data | for high Reynolds number | DEFINABLE ATTRIBUTE VALUE |
| 030b | Defined attribute 1 data | for low Reynolds number | DEFINABLE ATTRIBUTE VALUE |
| 030b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 031a | Defined attribute 2 name | circulating water | DEFINABLE ATTRIBUTE NAME |
| 031a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 031b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 032 | CONNECTION HEAD AND VALVES | SECTION TITLE |  |
| 033 | Connection head type | integral manifold head | CONNECTING HEAD |
| 033 | Connection head type | integral temp sensor | CONNECTING HEAD |
| 033 | Connection head type | integral valve head | CONNECTING HEAD |
| 033 | Connection head type | no manifold/head | CONNECTING HEAD |
| 033 | Connection head type | remote valve head | CONNECTING HEAD |
| 034 | Connection head style | horizontal parallel head | CONNECTING HEAD |
| 034 | Connection head style | horizontal/vertical head | CONNECTING HEAD |
| 034 | Connection head style | valve w/elbow fitting,tee | CONNECTING HEAD |
| 034 | Connection head style | valve with elbow fitting | CONNECTING HEAD |
| 034 | Connection head style | valve with nipple | CONNECTING HEAD |
| 034 | Connection head style | valve with nipple,tee | CONNECTING HEAD |
| 034 | Connection head style | vertical parallel head | CONNECTING HEAD |
| 035a | Process conn nominal size | 1 1/2 in | NOMINAL SIZE |
| 035a | Process conn nominal size | 1 in | NOMINAL SIZE |
| 035a | Process conn nominal size | 2 in | NOMINAL SIZE |
| 035a | Process conn nominal size | 3 in | NOMINAL SIZE |
| 035a | Process conn nominal size | DN 25 | NOMINAL SIZE |
| 035a | Process conn nominal size | DN 40 | NOMINAL SIZE |
| 035a | Process conn nominal size | DN 50 | NOMINAL SIZE |
| 035a | Process conn nominal size | DN 80 | NOMINAL SIZE |
| 035b | Process conn nom rating | cl 150 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | cl 1500 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | cl 2500 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | cl 300 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | cl 600 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | cl 900 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | PN 100 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | PN 16 | NOMINAL PRESSURE/RATING |
| 035b | Process conn nom rating | PN 40 | NOMINAL PRESSURE/RATING |
| 036a | Process conn termn type | flanged | END CONNECTION CATEGORY |
| 036a | Process conn termn type | hose | END CONNECTION CATEGORY |
| 036a | Process conn termn type | threaded | END CONNECTION CATEGORY |
| 036b | Process conn termn style | barbed | END CONNECTION TYPE |
| 036b | Process conn termn style | NPT(F) | END CONNECTION TYPE |
| 036b | Process conn termn style | NPT(M) | END CONNECTION TYPE |
| 036b | Process conn termn style | RF | END CONNECTION TYPE |
| 037a | Press conn nominal size | 1/2 in |  |
| 037a | Press conn nominal size | 1/4 in |  |
| 037b | Press conn termn style | NPT(F) |  |
| 037b | Press conn termn style | NPT(M) |  |
| 037b | Press conn termn style | SW(F) |  |
| 038 | Pressure tap orientation | angular |  |
| 038 | Pressure tap orientation | horizontal |  |
| 038 | Pressure tap orientation | vertical |  |
| 039 | Head orientation | axial,horizontal line |  |
| 039 | Head orientation | axial,vertical line |  |
| 039 | Head orientation | parallel,horizontal line |  |
| 039 | Head orientation | parallel,vertical line |  |
| 040 | Connection valve style | 3 valve manifold |  |
| 040 | Connection valve style | 5 valve manifold |  |
| 040 | Connection valve style | ball valve |  |
| 040 | Connection valve style | NA |  |
| 040 | Connection valve style | needle valve |  |
| 040 | Connection valve style | OS&Y gate valve |  |
| 041 | Head material | 316 SST | MATERIAL |
| 041 | Head material | carpenter 20 | MATERIAL |
| 041 | Head material | hastelloy C | MATERIAL |
| 041 | Head material | hastelloy C-276 | MATERIAL |
| 041 | Head material | monel | MATERIAL |
| 041 | Head material | monel | MATERIAL |
| 042 | Flange material | carbon steel | MATERIAL |
| 042 | Flange material | hastelloy C | MATERIAL |
| 042 | Flange material | hastelloy C-276 | MATERIAL |
| 042 | Flange material | monel | MATERIAL |
| 042 | Flange material | monel | MATERIAL |
| 042 | Flange material | stainless steel | MATERIAL |
| 043 | Connection valve matl | 316 SST | MATERIAL |
| 043 | Connection valve matl | brass | MATERIAL |
| 043 | Connection valve matl | bronze | MATERIAL |
| 043 | Connection valve matl | carbon steel | MATERIAL |
| 043 | Connection valve matl | hastelloy C | MATERIAL |
| 043 | Connection valve matl | monel | MATERIAL |
| 043 | Connection valve matl | monel | MATERIAL |
| 043 | Connection valve matl | NA | MATERIAL |
| 044a | Defined attribute 1 name | Blowout prevention type | DEFINABLE ATTRIBUTE NAME |
| 044a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 044b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 044b | Defined attribute 1 data | safety chain | DEFINABLE ATTRIBUTE VALUE |
| 045a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 045a | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 046a | Defined attribute 3 name | NA | DEFINABLE ATTRIBUTE NAME |
| 046b | Defined attribute 3 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 047 | INSERTION ASSEMBLY | SECTION TITLE |  |
| 048 | Insertion assembly type | gear drive |  |
| 048 | Insertion assembly type | hand crank drive |  |
| 048 | Insertion assembly type | handwheel drive |  |
| 048 | Insertion assembly type | manual drive |  |
| 048 | Insertion assembly type | NA |  |
| 049 | Isolation valve style | flanged ball valve |  |
| 049 | Isolation valve style | flanged gate valve |  |
| 049 | Isolation valve style | NA |  |
| 049 | Isolation valve style | threaded ball valve |  |
| 049 | Isolation valve style | threaded gate valve |  |
| 050a | Process conn nominal size | 1 1/2 in | NOMINAL SIZE |
| 050a | Process conn nominal size | 1 1/4 in | NOMINAL SIZE |
| 050a | Process conn nominal size | 2 in | NOMINAL SIZE |
| 050a | Process conn nominal size | 3 in | NOMINAL SIZE |
| 050a | Process conn nominal size | 3/4 in | NOMINAL SIZE |
| 050a | Process conn nominal size | DN 25 | NOMINAL SIZE |
| 050a | Process conn nominal size | NA | NOMINAL SIZE |
| 050b | Process conn nom rating | cl 150 | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | cl 300 | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | cl 600 | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | NA | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | PN 100 | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | PN 16 | NOMINAL PRESSURE/RATING |
| 050b | Process conn nom rating | PN 40 | NOMINAL PRESSURE/RATING |
| 051a | Process conn termn type | flanged | END CONNECTION CATEGORY |
| 051a | Process conn termn type | NA | END CONNECTION CATEGORY |
| 051a | Process conn termn type | threaded | END CONNECTION CATEGORY |
| 051b | Process conn termn style | NA | END CONNECTION TYPE |
| 051b | Process conn termn style | NPT(M) | END CONNECTION TYPE |
| 051b | Process conn termn style | RF | END CONNECTION TYPE |
| 052 | Valve nom press rating | 300 WOG |  |
| 052 | Valve nom press rating | 600 WOG |  |
| 052 | Valve nom press rating | cl 150 |  |
| 052 | Valve nom press rating | cl 300 |  |
| 052 | Valve nom press rating | NA |  |
| 053 | Valve body material | carbon steel | MATERIAL |
| 053 | Valve body material | NA | MATERIAL |
| 053 | Valve body material | stainless steel | MATERIAL |
| 054 | Valve seat material | NA | MATERIAL |
| 054 | Valve seat material | teflon | MATERIAL |
| 054 | Valve seat material | teflon | MATERIAL |
| 055 | Wetted material | carbon steel | MATERIAL |
| 055 | Wetted material | hastelloy C | MATERIAL |
| 055 | Wetted material | monel | MATERIAL |
| 055 | Wetted material | monel | MATERIAL |
| 055 | Wetted material | NA | MATERIAL |
| 055 | Wetted material | stainless steel | MATERIAL |
| 056 | Packing material | graphite | MATERIAL |
| 056 | Packing material | NA | MATERIAL |
| 056 | Packing material | nylon | MATERIAL |
| 056 | Packing material | teflon rings | MATERIAL |
| 056 | Packing material | urethane | MATERIAL |
| 056 | Packing material | zytel | MATERIAL |
| 057a | Defined attribute 1 name | Control rod material | DEFINABLE ATTRIBUTE NAME |
| 057a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 057b | Defined attribute 1 data | carbon steel | DEFINABLE ATTRIBUTE VALUE |
| 057b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 057b | Defined attribute 1 data | stainless steel | DEFINABLE ATTRIBUTE VALUE |
| 058a | Defined attribute 2 name | Control rod style | DEFINABLE ATTRIBUTE NAME |
| 058a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 058b | Defined attribute 2 data | dual rod | DEFINABLE ATTRIBUTE VALUE |
| 058b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 058b | Defined attribute 2 data | single rod | DEFINABLE ATTRIBUTE VALUE |
| 059a | Defined attribute 3 name | NA | DEFINABLE ATTRIBUTE NAME |
| 059b | Defined attribute 3 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 060 | PERFORMANCE CHARACTERISTICS | SECTION TITLE |  |
| 061a | Max press at design temp |  | PRESSURE |
| 061au | Design pressure units | psig | UNITS |
| 061b | Max design temperature |  | DESIGN TEMPERATURE |
| 061bu | Design temperature units | °C | UNITS |
| 061bu | Design temperature units | °F | UNITS |
| 061bu | Design temperature units | C | UNITS |
| 062a | Min working temperature |  | WORKING TEMPERATURE |
| 062au | Min working temp units | °C | UNITS |
| 062au | Min working temp units | °F | UNITS |
| 062au | Min working temp units | C | UNITS |
| 062au | Min working temp units | F | UNITS |
| 062b | Max working temperature |  | WORKING TEMPERATURE |
| 062bu | Max working temp units | °C | UNITS |
| 062bu | Max working temp units | °F | UNITS |
| 062bu | Max working temp units | C | UNITS |
| 062bu | Max working temp units | F | UNITS |
| 063a | Uncalibrated flow accuracy | 1 |  |
| 063b | Accuracy rating ref base | % flow coef | ACCURACY REFERENCE |
| 064 | Temp accuracy rating | 2 |  |
| 064u | Temp accuracy units | °C | UNITS |
| 064u | Temp accuracy units | °F | UNITS |
| 064u | Temp accuracy units | C | UNITS |
| 064u | Temp accuracy units | F | UNITS |
| 065a | Flow repeatability | 0.1 |  |
| 065b | Flow repeatability ref base | reading |  |
| 066 | Min Reynold's number | 10000 |  |
| 066 | Min Reynold's number | 12500 |  |
| 066 | Min Reynold's number | 15000 |  |
| 066 | Min Reynold's number | 2300 |  |
| 066 | Min Reynold's number | 25000 |  |
| 066 | Min Reynold's number | 4000 |  |
| 066 | Min Reynold's number | 6000 |  |
| 066 | Min Reynold's number | 6500 |  |
| 067 | Rated flow coefficient |  | RATED FLOW COEFF |
| 068a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 068b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 069a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 069b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 070a | Defined attribute 3 name | NA | DEFINABLE ATTRIBUTE NAME |
| 070b | Defined attribute 3 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 083 | ACCESSORIES | SECTION TITLE |  |
| 084 | Manifold valve style | 3-valve manifold |  |
| 084 | Manifold valve style | 5-valve manifold |  |
| 084 | Manifold valve style | NA |  |
| 085 | Manifold material | carbon steel | MATERIAL |
| 085 | Manifold material | Hastelloy C | MATERIAL |
| 085 | Manifold material | NA | MATERIAL |
| 085 | Manifold material | stainless steel | MATERIAL |
| 086a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 086b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 087a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 087b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 088a | Defined attribute 3 name | NA | DEFINABLE ATTRIBUTE NAME |
| 088b | Defined attribute 3 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 089a | Defined attribute 4 name | NA | DEFINABLE ATTRIBUTE NAME |
| 089b | Defined attribute 4 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 090 | SPECIAL REQUIREMENTS | SECTION TITLE |  |
| 091 | Custom tag | fiber tag |  |
| 091 | Custom tag | NA |  |
| 091 | Custom tag | sst permanent attached |  |
| 091 | Custom tag | sst wired-on |  |
| 092 | Reference specification | NA | REFERENCE SPECIFICATION |
| 093 | Special preparation | NA |  |
| 093 | Special preparation | special service |  |
| 094 | Compliance standard | NA |  |
| 094 | Compliance standard | NACE MR-01-75 | NACE STANDARD |
| 095 | Construction code | ANSI B31.1 |  |
| 095 | Construction code | ANSI B31.3 |  |
| 096 | Calibration report | lab flow calibration |  |
| 096 | Calibration report | NA |  |
| 096 | Calibration report | special calibration |  |
| 097 | Special inspections | dye penetrant ES-1012 |  |
| 097 | Special inspections | NA |  |
| 097 | Special inspections | radiographic examination |  |
| 098a | Defined attribute 1 name | Material reports | DEFINABLE ATTRIBUTE NAME |
| 098a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 098b | Defined attribute 1 data | ASTM traceable | DEFINABLE ATTRIBUTE VALUE |
| 098b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 099a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 099a | Defined attribute 2 name | Certification/Approval | DEFINABLE ATTRIBUTE NAME |
| 099b | Defined attribute 2 data | Canadian registration | DEFINABLE ATTRIBUTE VALUE |
| 099b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 100 | PHYSICAL DATA | SECTION TITLE |  |
| 101 | Estimated weight |  |  |
| 101u | Weight units | kg | UNITS |
| 101u | Weight units | lb | UNITS |
| 102 | Face-to-face dimension |  | FACE-TO-FACE DIMENSION |
| 102u | Face-to-face dim units | in | UNITS |
| 102u | Face-to-face dim units | mm | UNITS |
| 103 | Overall height |  |  |
| 103u | Overall height units | in | UNITS |
| 103u | Overall height units | mm | UNITS |
| 104 | Removal clearance |  |  |
| 104u | Removal clearance units | in | UNITS |
| 104u | Removal clearance units | mm | UNITS |
| 105 | Mfr reference dwg | NA |  |
| 105 | Mfr reference dwg | special product drawing |  |
| 106a | Defined attribute 1 name | NA | DEFINABLE ATTRIBUTE NAME |
| 106b | Defined attribute 1 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 107a | Defined attribute 2 name | NA | DEFINABLE ATTRIBUTE NAME |
| 107b | Defined attribute 2 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 108b | Defined attribute 3 name | NA | DEFINABLE ATTRIBUTE NAME |
| 108b | Defined attribute 3 data | NA | DEFINABLE ATTRIBUTE VALUE |
| 110 | CALIBRATIONS AND TEST | SECTION TITLE |  |
| 112a | Tag/Functional ident |  |  |
| 112b | Flow rate-Diff pressure | CALIBRATION/TEST DESC |  |
| 112c | Flow rate LRV | 0 | LOWER RANGE-VALUE |
| 112cu | Flow rate LRV units | ft3/min (std) | UNITS |
| 112cu | Flow rate LRV units | galUS/min | UNITS |
| 112cu | Flow rate LRV units | lb/h | UNITS |
| 112cu | Flow rate LRV units | m3/h (N) | UNITS |
| 112cu | Flow rate LRV units | m3/s | UNITS |
| 112cu | Flow rate LRV units | scfm | UNITS |
| 112d | Flow rate URV |  | UPPER RANGE-VALUE |
| 112du | Flow rate URV units | ft3/min (std) | UNITS |
| 112du | Flow rate URV units | galUS/min | UNITS |
| 112du | Flow rate URV units | lb/h | UNITS |
| 112du | Flow rate URV units | m3/h (N) | UNITS |
| 112du | Flow rate URV units | m3/s | UNITS |
| 112du | Flow rate URV units | scfm | UNITS |
| 112f | Diff pressure LRV | 0 | LOWER RANGE-VALUE |
| 112fu | Diff pressure LRV units | inH2O | UNITS |
| 112fu | Diff pressure LRV units | mmH2O | UNITS |
| 112fu | Diff pressure LRV units | Pa | UNITS |
| 112g | Diff pressure URV |  | UPPER RANGE-VALUE |
| 112gu | Diff pressure URV units | inH2O | UNITS |
| 112gu | Diff pressure URV units | mmH2O | UNITS |
| 112gu | Diff pressure URV units | Pa | UNITS |
| 113b | Test pressure | CALIBRATION/TEST DESC |  |
| 113d | Test pressure URV |  | UPPER RANGE-VALUE |
| 113du | Test pressure URV units | bar(g) | UNITS |
| 113du | Test pressure URV units | psig | UNITS |
| 118 | COMPONENT IDENTIFICATIONS | SECTION TITLE |  |
| 120a | Component type | averaging pitot tube |  |
| 120a | Component type | pitot tube |  |
| 120b | Manufacturer |  | MANUFACTURER |
| 120c | Model number |  | MODEL NUMBER |
| 121a | Component type |  |  |
| 121b | Manufacturer |  | MANUFACTURER |
| 121c | Model number |  | MODEL NUMBER |
| 122a | Component type |  |  |
| 122b | Manufacturer |  | MANUFACTURER |
| 122c | Model number |  | MODEL NUMBER |